[image: image1.jpg]4



[image: image2.png]




Glass Bead Sterilizer GS300L/GS300H

GS300 Series Glass Bead Sterilizer is ideal for the sterilization of surgical instruments (forceps, scissors, needles, etc.). When the chamber is filled with the included glass beads, high temperature(up to 300(C) can be used to eliminate the presence of bacteria, spores and other microorganisms. Without using potentially dangerous gases, flames or chemicals, it ensures that the sterilized instruments will be free of pathogens and microbial contaminants. The small footprint saves bench space, the sterilizer is safe to be used even in laminar flow hoods. 

Features:

Digital control, up to 300(C.
Sterilizes forceps, scissors, needles & other small surgical instruments(generally within 10-15s).

Safe to use—no gases, fumes, open flames or harsh liquids. 

Stainless steel inner container, built-in highly precise temperature controller and over-temperature protection.

Includes one bag of glass beads.

Specifications

	          Type

Parameter 
	GS300-L
	GS300-H

	Temperature range
	100(C~300(C

	Temp accuracy
	±5(C

	Display accuracy
	1(C

	Time range
	1min~99h59min/∞

	Heating time
	≤25min(RT. to 300(C)

	Voltage
	AC110V or 220V, 50/60Hz

	Material of lid
	Stainless steel

	Max power
	120W
	250W

	Chamber volume
(diameter x depth)
	Φ40mmx80mm
	Φ40mmx140mm

	Glass beads capacity
	150g
	300g

	Dimension(mm)
	176x135x189.5
	176x135x249.5

	Weight(kg)
	2.5
	3.3


www.ukaelab.com | sales@ukaelab.com

